This directory contains the code for a minikernel that is booted by

auxiliary microVAX processors configured on the same Q-bus backplane as

the UNIX host processor.

The kernel was written by Mike Parker at McGill University.  It is a

single process kernel, in that it only supports one distinct process

memory context. It does, however, provide asynchronous software

interrupts.  The system calls this kernel provides are enumerated

below.  They are used by RCI in creating and operating the RCI real

time control tasks.

Calls which are similar to those provided by UNIX:

brk(addr)

Set the high water mark of user memory to the indicated address.

Note that the sbrk() routine of UNIX is actually a library routine;

it is automatically supported as well.  But note that you probably

don't want to use sbrk(); see the getmem() call below for why not.

cmkrnl(fxn, arg)

Execute a function with the specified argument in kernel mode.  This

call is not present in standard 4.3BSD.

exit(code)

Stop execution of the user process.  The exit code is reported back

to the arbiter CPU if the arbiter has so requested.

getpagesize()

Returns the pagesize used by the kernel.

close(fd)

ioctl(fd, cmd, data)

read(fd, buffer, size)

write(fd, buffer, size)

fstat(fd, buf)

These calls emulate the close(), read(), write(), ioctl(), and fstat()

calls on UNIX, although the emulation is limited to the default file

descriptors 0, 1, and 2 (stdin, stdout, and stderr) which are in turn

hard-wired to the CPU console serial line.  The calls are provided so

that the stdio library can be used on the auxiliary CPU (though it

will work only when speaking to the default file descriptors, of

course: scanf(), printf(), getchar(), putchar(), etc).

Calls unique in character to the auxiliary kernel:

gettime100()

Returns the system time as measured by the 100 hz clock on the

microVAX CPU.

getmapplace(location)

Returns the address of various special memory locations that are

available for use by the process.  These locations are defined in

'getmapplace.h', and include:

MAP_TOY		-- address area of the Time of Year clock.

MAP_QBUS	-- address of the Q-bus.

MAP_IOPAGE	-- address of the Q-bus I/O page.

MAP_QBUSMAPREGS -- address of Q-bus mapping registers.

MAP_UAPAGE	-- a blackboard used for rapid, shared memory

communication between the kernel and the

process.  The functionality of this memory

area is described below.

MAP_BBOARD(cpu) -- the local virtual address the inter-CPU

communication blackboard for the indicated

cpu.  The functionality of this blackboard

is described below.  The argument cpu should

be 0 to refer to the arbiter CPU or 1, 2, or

3 for the auxiliary CPUs.

gettimesch()

Returns the time kept by the real time clock (if a real time clock is

available; otherwise the call will always return 0).

schslippage()

A debugging call that was used in determining (and fixing) the

problem with missed doorbell interrupts ... this call will return the

number of missed doorbell interrupts that have been detected in

association with the real time clock.  If you don't know about the

doorbell interrupt problem, don't worry about this system call.

sendsignal(cpu, sig)

Send a software signal to the indicated CPU.  If the destination CPU

is the UNIX arbiter CPU, then 'sig' will be interpreted as a UNIX

signal to be delivered to any process that has so requested such

delivery.  Signals can also be sent to auxiliary CPUs; software

running on the auxiliary CPUs can set up handlers for these (see the

handlesig() call below).

NOTE: The sendsignal() call currently works only when sending signals

to the arbiter or when sending to the CPU the call is being made on.

ast100(time, fxn, arg)

Calls the indicated 'fxn' with argument 'arg' as a software interrupt

when the system 100 Hz. timer reaches the indicated 'time'.  If the

time is less than or equal to the current time (as would be returned

by gettime100()), the software interrupt will be scheduled

immediately.

astsch(time, fxn, arg)

Calls the indicated 'fxn' with argument 'arg' as a software interrupt

when the system real time clock reaches the indicated 'time'.  As

with ast100(), if the time is past the interrupt is immediate.

getmem(size)

Grow the system memory area by 'size' bytes and return a pointer to

the allocated space. This is the equivalent of an ``sbrk()'' system

call.  We have to do this with a separate system call (instead of

using brk() directly, which is what sbrk() does on UNIX), because it

is also possible for the memory to be allocated on this CPU at the

request of the arbiter CPU.  Therefore, the information about where

the ``high water point'' of memory currently is must be kept in the

kernel.

clearast()

Reenable sofware interrupts.  Any software interrupts which were

scheduled but blocked will be delivered.

blockast()

Block all software interrupts (ASTs).  Any software interrupts which

are scheduled before the next clearast() will not happen until then.

handlesig(num, handler)

Causes the function 'handler' to be called as a soft interrupt when

the signal 'num' is received from another CPU (see 'sendsig()').

badaddr(addr, size)

Similar in function to the UNIX kernel level function 'badaddr':

attempts to access address 'addr' as a byte, word, or longword

reference (corresponding to a 'size' of 1, 2, or 4); the return value

is 0 if the reference succeeded and nonzero if it failed (caused a

machine check).

debugger()

Enters a very simple debugger utility, with interaction provided

through the auxiliary CPU console interface.  This debugger is

intended for debugging the auxiliary CPU's kernel, though it is also

possible to examine things in the user process on the auxiliary CPU

with it, if you know what you're doing.

halt()

Halts the cpu.

exportclock(cpu, code)

Used to broadcast the CPU's internal 100 Hz clock to another CPU,

thereby achieving a global wall clock.  The clock value will appear

as a value in the target CPU's inter-CPU blackboard.  The 'code'

describes how the exporting should be done:

0	-- don't export the clock.

1	-- export the clock only by updating the value in the

target CPU's blackboard.

2	-- update the value in the target CPU's blackboard, and

interrupt the CPU as well, so that it will be able

to trigger asynchronous events on the broadcast

clock.

Each CPU's clock value is always present in its own blackboard area.

schedast(cpu, time, fxn, arg)

Calls the indicated 'fxn' with argument 'arg' as a software interrupt

when the wallclock associated with the indicated 'cpu' reaches the

specified 'time'.  In order for the call to work, the indicated CPU

must be broadcasting its clock to the current CPU (see exportclock(),

above).

In addition to the system calls, there are two memory areas available

to provide shared memory (ie, fast) communication between various

pieces of software.  One area allows user-level code to communicate

with the kernel it's running under; the other is for inter-CPU

communication.  The addresses of these ``blackboards'' can be obtained

with the getmapplace() call (qv).

The local blackboard is called the user area page. The structure which

represents it is defined in "uapage.h".  The entire structure is

read/write to user code.  The fields include:

rt_handler	-- Writable variable which is a pointer to a

timeout handler function. Setting this to a

null pointer clears the timeout.  (This

variable is actually an int rather than a

pointer to function; cast the pointer to int

before assigning.)

rt_timeout	-- Writable variable which specifies the length

of time which should elapse before the

timeout fires.  The clock used is the built

in microVAX 100 Hz. clock.  Setting this to

1 or less will cause firing on the next

clock tick.

rt_argument	-- Writable variable which specifies the

argument which the timeout handler function

is to be called with.

block_asts	-- Writable variable which performs a similar

function to blockast(): setting it non-zero

masks all software interrupts.  This is made

available because the syscall mechanism (as

used by blockast()) takes a comparatively

long time, whereas writing a variable is

very fast.

astqsize	-- Variable describing the number of software

interrupts which are currently pending.

This variable is technically read/write, but

should never be written to.  Writing to it

won't break the kernel, but it may ruin its

usefulness, because it won't necessarily be

correct thereafter.  This is provided

because simply clearing the block_asts

variable is not sufficient to unblock ASTs

properly: the system doesn't hear about the

clearing immediately.  However, most of the

time there's nothing pending, and we don't

want to take the time for a syscall when

it's not necessary.  If this variable is

non-zero after clearing block_asts, the

clearast() syscall should be made to cause

the kernel to check for pending ASTs.

The inter-CPU blackboard is defined by the UNIX system file

/sys/local/kacomm.h.  Each CPU in the system (including the arbiter

CPU) has a blackboard that is exported onto the Q-bus.  It is a small

structure, that at the moment only contains the clock values that are

used by the CPU.  This area is read/write to user code as well, though

messing with the fields may cause software interrupts scheduled for the

future to be delivered at unexpected times if you don't know what

you're doing.  The fields are:

timesch		-- value of the real time clock.

time100		-- time of the built in 100 Hz. microVAX clock.

time_rt[4]	-- time value associated with a broadcast

clock.  If cpu 'n' is broadcasting a clock

to CPU 'k', then it will write the broadcast

clock value into the time_rt[n] of CPU k's

blackboard.  The array size of 4 reflects

the current limit of 4 microVAX CPU boards

on the Q-bus.  CPU 'n' will always keep

time_rt[n] in its own blackboard up-to-date.