RCI DEVICE DRIVERS vs. KERNEL DEVICE DRIVERS.

--- ------ ------- --- ------ ------ --------

This is intended to clear up some confusion that sometimes exists

about RCI device drivers. An RCI device driver is NOT a kernel device

driver. It is a small (and quite specific) set of functions with which

RCI does its own I/O without going through the kernel I/O system.

(The term "kernel" in this note refers to the kernel of the target

operating system, which is usually UNIX).

Understand what a device driver is: it is a package of routines for

doing I/O that can be called from an application program using a

generic function call interface.

RCI doesn't use the kernel I/O interface at really. The only place it

uses kernel device drivers is to provide interrupt handling to drive

control tasks running on the arbiter CPU. It has to do this because

the kernel controls the main handling of interrupts (we could get

around this with a good /dev/bus package, but that is for another day)

A diagram comparing RCI device drivers and UNIX kernel drivers may

be useful:

Regular UNIX program		RCI control task

|				|

V				V

UNIX kernel I/O routines		RCI I/O routines

read(), write(), etc.	    (prvate read() and write()

|		     functions.)

|				|

V				V

UNIX device driver		RCI device driver

|				|

V				V

/-----------------------------------------------------------------\

Bus

\-----------------------------------------------------------------/

As seen from the diagram, the device drivers have to have direct

access to the hardware bus. RCI has such direct access, through

the variables 'rciBus' and 'rciIOpage'.

'rciBus' accesses the main part of the bus, and 'rciIOpage' accesses

the smaller part of the bus that is reserved for I/O device registers.

Of course, what these two variables actually map onto is highly

dependant on the hardware of the particular system.

On a uVAX II, 'rciBus' pointers to the entire 22-bit Q-bus, and

'rciIOpage' points to the top 8K of the Q-bus which is reserved for

I/O registers.

On a SUN, 'rciBus' points to 4 MBytes of VME 24D16 space, starting

at offset 0x800000. We don't map the entire VME 24D16 space

because we simply don't have the room. 'rciIOpage' points

to the entire VME 16D16 space (which like the Qbus IOpage is generally

reserved for device control registers).

Why then does one have sometimes have to be concerned about kernel

device drivers with respect to RCI? Answer: If you are using a

hardware device interrupt to drive an RCI task, then you still have go

through the kernel in order to catch the interrupt. RCI has direct

access to the physical bus, and so we don't have to go through

the kernel to actually move bytes back and forth, but we don't have a

direct connection to interrupts. We still need a kernel driver to catch

the interrupt 	for us and pass it off to the RCI control code.

Since all an RCI kernel driver is doing is catching interrupts, it is

quite simple, and is also pretty much the same from device to

device. Have a look at the driver 'rci_template.c'

in the sys/local directory.