PRI ROBOT INTERFACE DRIVER

This document describes the parallel interface driver pri, which

is an RCI I/O device driver implemented using a DEC drv11 parallel

port. Because this is an RCI I/O device, it conforms to the generic

specification for RCI I/O devices (see IOdevice(7R)).

This driver was implemented to provide rapid packet oriented

communication between an RCI task on an VAX system and a special

interface program running on the LSI11 of a Unimation PUMA robot

controller.

FUNCTIONS CHARACTERISTICS:

The basic functions associated with this driver, and described below.

Most of the functions have timeout facilities built into them.

Since communication takes place over a parallel port, considerable

interaction is necessary between the driver routines at both ends.

RCI I/O devices maintain two types of timeout parameters. The first

is dev->blocktime, which specifies how long a routine which is

expected to block should block before giving up. The second is

dev->timeout, which defines the timeout time associated with

immediate interactions.

priRead (dev, buf, size)

Reads in a packet of bytes from the target device into a

buffer specified by 'buf' and 'size'.

The routine blocks until the target device is ready to write.

The blocking time is limited to 'blocktime' milliseconds,

where 'blocktime' is a field in the 'dev' structure which can

be set using the IOC_SET_BLOCKTIME ioctl.

The routine returns the number of bytes that were read,

or -1 if an error occurred.

Error codes are returned in dev->error, and include:

ECount --    incoming packet specified a byte count less

than 0 or greater than 'size'.

ETimeout --  timeout occurred while blocking or during

transmission.

ETransmit -- A checksum error was detected.

priWrite (dev, buf, size)

Writes a buffer 'buf' of 'size' bytes to the target device.

The routine blocks until the target device device is ready

to read. The blocking time is limited to 'blocktime'

milliseconds, where 'blocktime' is a field in the 'dev'

structure which can be set using the IOC_SET_BLOCKTIME ioctl.

The routine returns either number of bytes that were written

(size), or -1 in the event of an error.

Error codes are returned in dev->error, and include:

ECount --    specified number of bytes was less than 0.

ETimeout --  timeout occurred while blocking or during

transmission.

priProbe (dev)

Called to check to see if the device is present at its

indicated address, and if so, to translate the address

into a memory location that can be used to access the device

registers directly.

The address is passed into the probe routine in the field

dev->address, which is set up when the device is allocated.

The address is interpreted as a 16 bit Q-bus address, and

the low 13 bits of this are used to define the address in

the Q-bus I/O page. The probe routine finds the memory

location of the device by adding this low 13 bits to the

base address of the memory mapped I/O page (rciIOpage).

The probe routine also sets the 'timeout' and 'blocktime'

parameters to default values, instantiates dev->size to the

size of the device register map (in bytes), and returns this

size.

In the event of a non-existent device, the probe routine

returns -1 and sets dev->error to ENoDev.

priAttach (dev)

The purpose of this routine is to establish a connection to

the target device. The meaning of this is actually a bit ill

defined. In practice, it simply means it communicates with the

target device to ensure that the target device is operational

and is capable of receiving I/O. It does this by signaling the

target device and doing a complete handshake with it.

Correspondingly, it also serves as a signal to the target

device that a set of I/O sequences is about to begin.

Given the more recently implemented signal mechanism, this

routine could probably be removed, with its function being

replaced by a priSignal() call. Using a separate call to do

the attach probably makes the protocol a bit more robust.

The way the routine works is to first check to see that the

target device is listening for an attach. It then asserts a

listen state itself and sends an interrupt to the target

device, after which it waits for the device to respond by

dropping the listen state. The driver then drops its

listen state and waits for a final response from the target

device.

The routine returns 1 if successful, or -1 in the event of an

error. Error codes are set in dev->error, and include:

EConnect --	Target device was not listening for a connect.

ETimeout --	Timeout occurred while talking while doing the

attaching handshake.

priDetach (dev)

Shuts down the device. This does not actually a communication

session with the target device in that no communication with

the target device takes place. Rather, the routine simply

disables interrupts and clears all device registers.

The routine always returns 1.

priSignal (dev, n)

Send a signal number 'n' to the target device.

This routine delivers an interrupt signal to the target

device, which may have set up a handler for the signal.

The routines works by using one of interrupt lines on the

parallel port to get the host device's attention. It then some

simple communication with the interrupt service routine to

convey the signal number.

The routine returns 1 if the signal was delivered

successfully, and -1 otherwise. Error codes are placed in

dev->error, and include:

ETimeout --	Timeout waiting for the target device to

respond.

ECount --	The signal number was out of range. The legal

signals at the time of this writing are 1 and

2.

ESync --	Protocol error.

priIoctl (dev, com, data)

I/O control interface for the driver. This allows software to

set driver parameters. All of the driver ioctls defined in

IOdevice(7R) are supported, including:

IOC_SIGINTER_ENABLE --

Enables interrupt A of the parallel port.

IOC_SIGINTER_DISABLE --

Disables interrupt A of the parallel port.

IOC_TIMEOUT --

Sets dev->timeout.

IOC_GET_BLOCKTIME --

Returns dev->blocktime.

IOC_SET_BLOCKTIME --

Sets dev->blocktime.

The routine returns 1 unless a bad command is specified, in

which case -1 is returned and dev->error is set to EBadIoctl.

priTestRead (dev)

Returns 1 if the target device is ready to write (i.e.,

a call to priRead() would not block), and 0 otherwise.

priTestWrite (dev)

Returns 1 if the target device is ready to read (i.e,

a call to priWrite() would not block), and 0 otherwise.

USE OF THE DRIVER WITH THE EXISTING LSI11 PROGRAM 'MOPER'.

The LSI11 program 'moper' that this driver was mainly designed to work

with has a package of routines that interface to this driver. These

are known as the 'host I/O' routines. How these routines interact with

the pri driver is described here.

The host I/O routines are defined in the 'lsi11' directory. They

include:

hostListen(fxn, arg)

Sets the program listening for an attach request, and

arranges for the function 'fxn' to be called with argument

'arg' when the attach request is actually received. The attach

request is invoked from the host by the priAttach() function.

The function 'fxn' must quickly complete the attach handshake

by calling hostAttach().

hostAttach()

Completes the attach protocol that is started when the host

calls priAttach() and moper's attach handler is called. The

attach handler is set up earlier by moper with a call to

hostListen().

hostOnSig(signum, fxn, arg)

Sets up a the function 'fxn' with argument 'arg' as a handler

to be called when signal 'signum' is sent from the host with

the priSignal() routine.

The signal handler will only be called once. For additional

calls, the handler must be set up again using this routine.

hostDetach()

Disbales all interrupts and puts the parallel port into a

known state by clearing all registers.

hostRead(buf, size)

Reads a packet of words from the host into 'buf', where the

packet may be up to 'size' words long (Note that words

correspond to 2 bytes on 'moper'). The routine waits until the

host is ready to write a packet of words with priWrite().

hostWrite(buf, size)

Writes a buffer of 'size' words to the host. The routine waits

until the host is ready to read a packet of words with

priRead().

hostSignal()

Asserts that 'moper' is ready to write data to the host (or

will be very shortly), and also sends and interrupt to the

host. This interrupt serves only as the polling interrupt if

the RCI task that moper is communicating with is running on

the arbiter and requires a polling interrupt.

The typical interaction of the routines on both the host side and

moper side during a typical RCI robot control cycle is shown below.

RCI TASK SIDE			MOPER SIDE

Set up phase:

priProbe(d)			hostListen(attachHandler, 0)

Attach phase:

priAttach(d)	--------------> invokes attachHandler ...

hostOnSig(2, release, 0)

hostAttach() ... completes attachment

priRead(d,b,s)	<-------------- hostWrite(b,s)

priWrite(d,b,s) --------------> hostRead(b,s)

hostOnSig(1, control, 0)

Control phase (moper slave mode):

priSignal(d,1) 	--------------> invokes control function ...

priTestRead(d)	<-------------- hostSignal()

priRead(d)	--------------> hostWrite(b,s)

priWrite(d) 	<--------------	hostRead(b,s)

priSignal(d,1) 	--------------> invokes control function ...

priTestRead(d)	<-------------- hostSignal()

priRead(d)	--------------> hostWrite(b,s)

priWrite(d) 	<--------------	hostRead(b,s)

.

.

.

Release phase:

priSignal(d,2)	--------------> invokes release function ...

priWrite(d,b,s) <--------------	hostRead(b,s)

priRead(d,b,s)  --------------> hostWrite(b,s)

This example assumed that the communication between the host and

moper takes place in 'slave' mode, where the host drives moper.

The only difference for master mode is that moper control function

wakes up not off of a signal from the host, but from some internal

clock signal. The line

priSignal(d,1) 	--------------> invokes control function ...

is hence omitted.

USING DRIVERTEST TO CHECK OUT THE PRI DRIVER.

The program 'driverTest', in this directory, can be used to test

RCI device drivers. The program creates a task off of which the user

can attach a device at a particular address. The various driver

routines can then be called interactively, assumably in conjunction

with a program on the target device side which is executing the

corresponding target driver routines.

The program 'testhostIO' in the 'lsi11' directory is an exerciser for

the 'host I/O' functions of moper. It can be downloaded to an LSI11

robot controller and executed by itself.

To test the pri driver - moper interface, then, both the driverTest

program and the testhostIO program should be started up. Since in

testing the interface, the RCI task may occasionally block for long

periods of time, this will freeze the VAX if the RCI task in

'driverTest' is created on the arbiter CPU. If the VAX is frozen, and

communication with testhostIO is being done through termlink, then

this communication will freeze also. The easiest solution

to this is to create the "driverTest" control task on another

cpu; this can be done by starting driverTest with the option

"-cpu <n>". If no auxiliary CPU is available, then a terminal

should be connected to the LSI11 console line directly (instead

of communicating using termlink) so that characters will not have

to be passed through the VAX. This can only be done, however, once

testhostIO has been downloaded from the VAX.

Assuming that both programs are now operational, the first thing

that should be done is to attach the desired device to 'driverTest'.

DRIVERTEST> set device pri eff8

device size is 6

DRIVERTEST>

Next, the 'detach' routine should be called on both ends.

This simply puts each interface into a known state. The detach should

return successfully in both cases.

DRIVERTEST> detach		HOSTIO> d

1

Now the listen/attach sequence should be tested.

HOSTIO> l

DRIVERTEST> attach		HOSTIO>

1				host attached, return value 1

The attach was successful. We can now test read and write

sequences. Before doing that, the default block time should be

set to several seconds in 'driverTest' to allow time to type an I/O

command into testhostIO. 10 seconds is probably good enough.

DRIVERTEST> ioctl SET_BLOCKTIME 10000

1

Now we read and write 6 bytes from either side. Remember that

testhostIO asks the user for the number of words (2 bytes) to write.

We will also test the testRead and testWrite routines on the host

at the same time.

Testing read from host:

DRIVERTEST> read

HOSTIO> w

number of words> 3

6				3

0 01 00

2 03 02

4 05 04

DRIVERTEST> testRead

0

DRIVERTEST>

HOSTIO> w

number of words> 3

DRIVERTEST> testRead

1

DRIVERTEST> read

6				3

0  01 00			HOSTIO>

2  03 02

4  05 04

Testing write from host:

DRIVERTEST> write 6

HOSTIO> r

6				3

0  01 00

2  03 02

4  05 04

HOSTIO>

DRIVERTEST> testWrite

0

DRIVERTEST>

HOSTIO> r

DRIVERTEST> testWrite

1

DRIVERTEST> write 6

6				3

DRIVERTEST>			 0  01 00

2  03 02

4  05 04

We now test the signal routines. On the testhostIO side the command

'o' sets up a signal handler for a particular signal that prints a

message.

Check to make sure nothing happens if no handler is set up.

DRIVERTEST> signal 1		HOSTIO>

1

DRIVERTEST> signal 2

1

DRIVERTEST>			HOSTIO> o

signal number> 1

DRIVERTEST> signal 1		HOSTIO>

1				caught signal 1

Check to see that sending the signal again does nothing.

DRIVERTEST> signal 1

1

DRIVERTEST>

Now handle the second signal.

o

signal number> 2

DRIVERTEST> signal 1		HOSTIO>

1

DRIVERTEST> signal 2

1				caught signal 2

DRIVERTEST>

Finally, we check out the routine hostSignal. We first enbale

interrupts on the host side with an ioctl. Sending the signal

should then cause a stray interrupt message to appear on the console,

since the RCI device is not operational. We should also see the

WRITE_READY signature value in the parallel port input buffer

on the host.

DRIVERTEST> ioctl SIGINTER_ENABLE

1

DRIVERTEST>			HOSTIO> s

<stray interrupt message on console>

DRIVERTEST> examine

0  0x00d0

2  0x1111

4  0x0000

DRIVERTEST>