And now a few notes on interpreting stack trace backs on

microVAX RCI systems.

I ran an actual example, which we can go over here. I forced

a divide by zero fault in a program, and got the following stack

trace:

Program Crash on CPU 1

arithmetic trap, pc = 0xbf26 (floating divide by zero fault)

Stack trace follows --

PC values  arguments

bf26 :      4190        0        0        0

bf03 :         1

bd06 :     22d4c        0

c0cb :     22d4c

bf7e :     31b38

1229f :         1

The numbers on the left are program counter values, and the

numbers on the right are the arguments to the routine which

is currently executing. These arguments are displayed as

32-bit words, so you have to be careful in interpreting them

if the passed arguments to the routine are a different size

than 32 bits (for instance, float and double, are both

passed as 64 bit words).  The order of nesting is

innermost-on-top, that is, in the above example the crash

occurred at bf26, which was called from bf03, which was in

turn called from bd06, etc.

To make sense out of the stack trace, you need to use adb. The

session at which I did this is given here:

Curly-rci.new> adb rcitest

0bf26 ? i

_divide+4:      divd2   c(ap),4(ap)

4190 = f

+4.5000000000000000e+00

0 = f

+0.0000000000000000e+00

0bf03 ? i

_testCrash+1f:  brb     _testCrash+3a

0bd06 ? i

_cfstart+88:    movl    -4(fp),r0

0c0cb ? i

_taskCheckOut+53:               tstl    r0

(Notice that when a hexadecimal number begins with a letter, we

must type a leading zero; otherwise adb tries to interpret it

as a symbol name.)  The first thing I do is find the location

where the fault occurred.  This is the first pc value in the

stack trace. Sure enough, when we disassemble it, it gives us a

divide instruction. The routine is '_divide' (remember that the

C compiler puts and extra _ on the front of its symbols, so in

the C program this would be only called 'divide'). I want to

see what the arguments to divide() are, so I look at the

argument list. There are 4 long words there, but since divide

takes 2 floats as an arguments (which are converted to double

during the subroutine call), I know these 4 long words really

represent 2 double precision numbers. To convert from the hex

code to a floating format, I can type "<number> = f" in adb,

and it does the conversion for me. Although two hex numbers

represent each double, I only worry about the first number of

each pair, since all the significant bits and the exponent are

contained there; the second number is just extra precision

(which is 0 in this case anyway). The second argument to divide

is 0.0, because I know that the hex code for a double 0.0 is in

fact (0, 0), but just to illustrate the point I convert it as

well.

To find out what routines called 'divide()', I proceed further

with the stack trace. All other PC values in the stack trace

represent the instruction immediately following the call to the

routine below it. By looking at the location of each PC value,

I can figure out what routines called what. For instance, 0bf03

is at _testCrash+1f, so I know 'divide()' was called from

'testCrash()'.  The argument to 'testCrash()' was 1 (since

testCrash() takes an integer argument, the stack trace gives

its value (in hex) explicitly).  Similarly, I find that

'testCrash()' was called from 'cfstart()', which has two

arguments: 22d4c and 0. The first is a pointer, and the second

is a integer (again, I know this by looking at the definition

of 'cfstart()'.

Sometimes, adb will not want to give you the symbol-relative

value of an address. This will happen if you are more than 256

bytes away from the beginning of the routine. In that case,

force adb to give you a symbol-relative location by typing the

command

0x7ff $ s

into adb.  This will not produce any output; you should repeat

the adb command which didn't give you a symbol-relative

printout, and now it should produce a more useful result.

Finally, a word about pointers. Many of your crashes will be

attributable to bad pointers, so it is a good idea to be able

to tell if a pointer is OK or not by looking at it.  A pointer

with a value from 0 to the end of the text segment will usually

be bogus, since it points into code (unless the pointer is a

pointer to a function, in which case it SHOULD be between 0 and

the end of the text segment). A 0 pointer is invariably bad,

since even if the pointer is a function pointer, executing it

will simply cause the program to start again.  Data pointers

should either be in the data segement, or close to it (the

effective data area consists of the data and bss segments, and

then the adjacent region which grows dynamically when we call

malloc()), or on the stack. The stack starts at 0x80000000 and

grows down, so a typical stack pointer value is 0x7ffff800.

Stack pointer values below 1 Meg are almost certainly out of

bounds, as are values greater than 1 Meg. Hence reasonable

pointer values are from then end of text tup to 0x100000, and

from 0x7fffffff down to 0x7ff00000. To find out the size of the

text segment (and the other segments for that matter), run the

program 'size' on the executable image.

The only exception to these ranges is when you have a control

task executing on the arbiter. In that case, the stack used is

the interrupt stack, which should live somewhere in system

space between 0x80000000 and, say, 0x80200000.