This file documents the functionality provided by the LSI11

microkernel. The microkernel is contained in code found in the

following files:

locore.c

kernel.c

serialDev.c

serialDev.h

sys.h

exit.c

lsi11_73.h

To an application program, the microkernel offers the following

system services:

Basic untility routines ---

spl7()

Sets the interrupt priority to 7, and returns the old

interrupt priority. Priority 7 is the highest priority,

which masks all device and clock interrupts. It is mainly

used to guard critical sections.

splx(s)

int s;

Sets the interrupt priority to the level s, and returns

the previous priority. splx() and spl7() are often used

in tandem to do a mask and unmask interrupt sequence:

s = spl7();

... critical code ...

splx(s);

wait()

Puts the processor into a state where it waits for the next

bus event (device of clock interrupt). This call should be

used with caution on some of the Unimation robot controllers,

because there is special hardware that causes the emergency

stop condition to be set if the processor is idle for more

than a few milliseconds. In these cases, we have defined

wait() as a busy loop macro instead.

halt()

Halts the processor, so that it enters the ODT monitor.

restart()

Restarts the program from 'square one', resetting all

interrupt tables and the like. It does not, however,

rezero the bss segment, and of course data in the data

segment may have any old value.

setjmp(buf)

int buf[10];

The standard set up for longjmp() (see below). Setjmp()

and longjmp() implement a catch and through. Setjmp()

sets up a catch frame in 'buf', saving the current register

state and stack frame. It then returns 0.

longjmp(buf, code)

int buf[10];

int code;

The counterpart to setjmp() (see above).

When a call is made to longjmp() from a point in the code

at the same level as setjmp(), or nested deeper down, the

call frame and register context is restored to that which was

saved by setjmp() inside 'buf'. The routine then returns

'code'. To the program, it looks like setjmp() has returned

again, this time with 'code' as a return value.

Setjmp() and longjmp() are fairly standard C library routines,

and are basically used to implement a goto across procedures

that allows a program to quickly bail out of a mess or

exceptional condition.

Clock and timer AST routines ---

long gettime()

Returns the system time value (which is a long word containing

the number of clock ticks that have elapsed since that

program was first loaded.

clkast (fxn, arg, count)

int (*fxn)();

int arg;

int count;

Puts a request in to the system to call the specified function

'fxn' (with the indicated argument 'arg') after 'count'

clock ticks have elapsed. If 'count' is less than or equal

to zero, then the function will be called on the next clock

tick. If the 'fxn' is NULL, then any pending clock AST is

cancelled. The routine returns the function address of any

previously pending clock AST.

The clock AST routine is called at maximum priority (spl7())

with all registers saved.

timeout (fxn, arg, ticks)

int (*fxn)();

int arg;

int count;

This puts in a request for a timeout AST, which is really

just another clock AST with a different name.

The system is requested to call the specified function

'fxn' (with the indicated argument 'arg') after 'count'

clock ticks have elapsed. If 'count' is less than or equal

to zero, then the function will be called on the next clock

tick. If the 'fxn' is NULL, then any pending timeout AST is

cancelled. The routine returns the function address of any

previously pending timeout AST.

The timeout routine is called at maximum priority (spl7())

with all registers saved.

getmem (size)

int size;

Memory allocator routine. Returns a block of memory

(word aligned) which is 'size' bytes long. If no more memory

is available, the routine returns NULL.

Register saving and restoring ---

pushr()

Saves all current registers on the stack, from which they can

be restored with popr(). This routine is mainly intended for

interrupt service routines which are doing their own register

saving.

popr()

Restores registers from the stack, to which they were

(assumably) previously stored by pushr().

Interrupt vector attachment ---

VECTOR *vectorAlloc (vecnum, vdesc)

unsigned vecnum;

VECT_DESC *vdesc;

Attaches an interrupt service routine to vector number

'vecnum'. The service routine and the type of attachment

is described by the contents of the descriptor 'vdesc'.

The system sets up the interrupt vector so the routine

vdesc->fxn is called with argument vdesc->arg whenever

an interrupt occurs at the specified vector. If

vdesc->saveregs is true, then the system will save all

registers before calling the routine. Otherwise, it be up to

the routine to save registers. The latter may be preferred for

speed. The routine is called at maximum priority (spl7()).

Priority can be dropped within the routine using splx().

The routine returns a pointer to the dispatch block, although

this feature is a bit historical. The programmer can reassign

the service routine by reaching directly into the dispatch

block, although this is a bit rugged; it is easier to

simply call allocVector() again.

Serial line interface routines ---

sluFlush (slu)

SLU *slu;

Clears all read and write buffers attached to the serial

line.

sluInit (num, unitDesc)

int num;

DEV_DESC *unitDesc;

Called at program startup time to initialize the serial

line units that can be controlled using the microkernel

serial line unit package.

Takes a table of descriptors ('unitDesc') that is 'num'

entries long and configures a serial line unit for each

entry in the table.

The program can later obtain the serial line unit descriptor

by calling getSlu() with the unit number.

SLU *sluGet(num)

int num;

Returns a serial line descriptor for unit number 'num', where

num is an index into the table that was passed as an argument

to 'sluInit()'.

sluOn (slu)

SLU *slu;

Turns the serial line unit 'on' by enabling interrupts. The

routine also clears all read and write buffers.

sluOff (slu)

Disbales interrupts for the indicated serial line unit, and

clears all read and write buffers.

sluRead (slu, buf, size)

SLU *slu;

char *buf;

int size;

Sets up a read buffer 'buf' of 'size' bytes for the serial

line unit. After this call, all characters received by the

serial line will be placed in 'buf', until buf is filled

up, the serial line is turned off, or another call is made

to sluRead().

sluWrite (slu, buf, size)

SLU *slu;

char *buf;

int size;

Tells the serial line unit to output the buffer 'buf' of

'size' bytes. The serial line will continue to do this until

the buffer is empty, the serial line is turned off, or another

call is made to sluWrite().

sluRcount (slu)

SLU *slu;

Returns the number of characters that have been read by the

serial line since the last call to sluRead().

sluWcount (slu)

SLU *slu;

Returns the number of characters that have been written by the

serial line unit since the last call to sluWrite().