\section{Real-time Installation for VME-based Hosts with PUMA Robots}

This encompasses VME-based hosts (Sun4 or SGI IRIS)

with connections to PUMA robots controlled by Mark I, II,

or III controllers. Each robot controller is assumed to be connected

to the host system by a DEC DRV11 and a XYCOM XVME 240.

The hardware aspects of the installation are detailed

in the {\it RCCL/RCI Hardware Installation Notes}.

\section{Hardware}

\begin{enumerate}

\item

{\it Equipment needed --}

You need a robot, a controller, and a host system. For each robot controller,

you need a DEC DRV11 and a XYCOM XVME 240. If you want hardware clock

support on the Sun system, you also need a XYCOM XVME 203 or 293.

S-bus based Suns should be attached to a VME card cage using a

PT-SBS915 S-bus to VME adaptor, {\it with} configuration software.

\item

{\it Cable interface --}

Build a cable interface to connect each DRV11 and XVME 240.

\item

{\it Hookup --}

Jumper each XVME 240 appropriately and plug it into the host system

VME bus. Jumper each DRV11 and plug it into the Unimate controller

Q-bus. Connect the two boards via the cable connector. Attach

the console serial line from the robot controller to a serial port

on the host system. If you have an XVME 203 or 293 for the Sun system,

jumper this and plug it into the VME bus.

\item

{\it Controller customization --}

Upgrade the LSI11 to an LSI11-73, if necessary.

Wire up the ``auto power on'' circuit, if you desire this feature.

\end{enumerate}

\section{Software}

\begin{enumerate}

\item

{\it Build RCCL distribution --}

See Section \ref{installation.sec}.

Unpack the tape, install GNU {\tt make}, edit the file {\ff conf/site.conf},

and run ``{\tt make World}''. Install the simulator network service.

\item

{\it Install kernel support, if necessary --}

On Sun4 or SGI systems, add kernel support as described in either section

\ref{Sun4kern.sec} or \ref{SGIkern.sec}.

Include the

{\tt genrci} drivers, the {\tt xyd} driver, and (for Sun4 systems

with a XVME 203 or 293) the {\tt xyclk} driver. Create special files

for the drivers in {\ff /dev}. Verify RCI installation with

{\tt rciCheck} (Section \ref{rciCheck.sec}).

\item

{\it Configure controller serial line connection --}

See the {\it RCCL/RCI Hardware Installation Notes}.

Locate the file in {\ff /dev} that controls the serial device

connected to the robot controller. Verify the connection with

{\tt termlink}.

\item

{\it Configure RCI for each robot --}

For each robot, give it a name and an entry in

{\ff robots/robotDataTab.c} (Section \ref{robotdatatab.sec})

and {\ff conf/robots}

(Section \ref{robotsfile.sec}).

Create a {\tt .jls}, {\tt .kyn}, and

{\tt .pos} file for each robot

(sections \ref{jlspuma.sec}, \ref{kynpuma.sec}, and \ref{posfile.sec}).

\item

{\it Configure {\tt moper} for each robot --}

For each robot, create a {\tt moper} for it in the {\tt lsi11}

directory, and adjust its configuration file

(Section \ref{newmoper.sec}). Build the

new {\tt moper} and load it into the robot controller over the

serial line.

\item

{\it Test the robot connections --}

With {\tt moper} running in your controller, run the

test programs {\tt rbtcom} and {\tt primecal} as described

in Section \ref{testing.sec}.

\item

{\it Calibrate the robots --}

Calibrate each robot using {\tt primecal} and {\tt potcal} as

described in sections \ref{primecal} and \ref{potcal}. If you want to

use current control programs such as {\tt zerograv}, then

do the calibrations described in Section \ref{curcalib.sec} and

\ref{gravcal.sec}

as well.

\end{enumerate}