RCCL/RCI Local Guide

--------------------

LCI lab, Computer Science, UBC

------------------------------

John E. Lloyd

Feb 11, 1999

The RCCL root directory:

At the time of this writing, RCCL/RCI lives in ~lloyd/rccl/rccl.5.0,

although this should probably be moved over to somewhere in

/lci/local,

The environment variable RCCL should be set to point to the RCCL

root directory.

Documentation:

Fairly extensive documentation is available in the directory

$RCCL/doc.

For documentation on RCCL itself, see the "RCCL User's Guide"

(doc/rcclGuide/rcclguide.ps)

For documentation on starting up and installing the system,

see the "RCCL/RCI Startup and Installation Guide"

(doc/installGuide/install.ps)

Setting up the RCCL environment:

For information on setting up the RCCL environment and compiling

RCCL programs, see the "RCCL/RCI Startup and Installation Guide",

Sections 1 through 2.2.

If you are running the CRS, you will need to run your

programs on zima, for which the target system name is "Sun4sol".

If you are running the Puma 260 or any of the acme devices,

you will need to run your programs on kiewa, for which the target

system name is "X86sol".

Also, for controlling the PUMA 260, set up the following aliases:

alias load260 \

'down -d /dev/ttyd1 -e 10000 -a 10000 $RCCL/lib/generic/moper/thumper -t'

alias talk260 termlink -d /dev/ttyd1

Powering up and calibrating the CRS:

1. Power up the CRS controller (rotary switch)

2. Turn on the arm power (push button switch)

3. Make sure that the arm is completely clear for calibration.

This means moving the table out of the way.

4. Calibrate (or "home") the arm by hitting F1 on the teach

pendant. The arm will seek out its home position and

calibrate itself.

Enabling the RCCL/RCI interface for the CRS:

1. If zima has been rebooted recently, you will need to enable

real-time opeartion on the system by running /usr/local/rtpatch.

2. Load the communication software into the transputer

linking the CRS with zima:

% rciTspLoad

3. To verify that RCCL/RCI can communicate with the robot,

run the program

% free

which should produce the prompt

FREE>

Typing show at the prompt should give a list of the present

joint angles.

4. Move the arm to some desired position. This can be some

using the command

% move to <positionName>

where position name is a joint-space position defined in

$RCCL/lib/generic/robotData/caesar.pos

You can also specify joint angles to 'move' explicitly;

see "man -s 1 move"

(Note that the RCCL/RCI name for the CRS installed in LCI is

caesar)

Powering up and calibrating the Puma 260:

The robot is controlled from kiewa.

1. Turn on the robot controller (large toggle switch in front)

2. Put the controller into ODT by toggling the "run/halt" switch

located along the top edge of the controller (i.e., setting

the switch to the "halt" position and then back into the

"run" position).

3. Load the "moper" control program into the controller with

the command

% load260

This should take a couple of minutes. If moper loads

successfully, the teach pendant should beap a couple of

times.

4. If the robot is in a known position (like rcclpark),

calibrate the robot by running the command

% pumacal -at <position>

where <position> is a joint-space position defined in

$RCCL/lib/generic/robotData/thumper.pos.

If the robot is *not* in (or within a couple of degrees)

of a know position, you will need to put it in the

nest. To do this, first free the robot, using the command sequence

% free

FREE> free all

Make sure the arm is supported before freeing it.

Type <CR>, or hit the 'FREE' button on the pendant, to confirm:

Once the robot is in the nest, run the command

% pumacal

(if there are no arguments, pumacal assumes the robot

is in the nest).