From michel@ireq-robot.hydro.qc.ca Wed Aug 30 06:17:49 1995

Received: from amadeus.ireq-robot.hydro.qc.ca (amadeus.ireq-robot.hydro.qc.ca [131.195.20.131]) by grolsch.cs.ubc.ca (8.6.10/8.6.9) with SMTP id GAA19108 for <lloyd@cs.ubc.ca>; Wed, 30 Aug 1995 06:17:40 -0700

Received: from borduas.ireq-robot (borduas.ireq-robot.hydro.qc.ca) by amadeus.ireq-robot.hydro.qc.ca (4.1/IRBT-2.23)

id AA21608; Wed, 30 Aug 95 09:17:32 EDT

Date: Wed, 30 Aug 95 09:17:32 EDT

From: Michel Pelletier <michel@ireq-robot.hydro.qc.ca>

Message-Id: <9508301317.AA21608@amadeus.ireq-robot.hydro.qc.ca>

To: lloyd@cs.ubc.ca

Subject: Re: Impedance control revisited

Status: RO

Content-Length: 7777

Hi again John,

This is a very interesting question (for me at least!), and I hope

I can provide you with useful info.   Here goes:

Since you are looking at that particular paper, I'm assuming you are

looking for an impedance controller using a force sensor and a

position-controlled robot.  In the paper I present four algorithms,

and as I say, the one we use because it is simple, the most stable,

and has the best tracking performance (with respect to the nominal

impedance equation) is the velocity method with a velocity correction

factor.  I don't use the acceleration feedback term.

Another reason for using that method is that Cartool (my robot

programming environment, built originally as a user-interface on top

of Kali) actually blends all inputs (position, velocity and force

demands) as position increments (which can also be seen as velocities)

and adds them together to create a global displacement during 1

iteration.  The velocity method is directly compatible with this.  The

idea of this was to be able to perform many types of compliant motions:

in the task frame, selected by the user, I can send position

trajectories, velocities and/or forces, independently in each

direction (along each axis), so I can create hybrid motions, shared

control (when the velocity inputs come from a hand controller in some

directions), and guarded moves very easily, while always having the

underlying impedance controller to limit contact forces.  The

trajectory generation in Cartool (which replace Kali's) is based on

this, and in free space, position trajectories are simply sent in all

6 directions to create straight-line motions.  (Cartool also has other

neat stuff: relative/absolute motions, automatic singularity crossing,

setpoints definitions, and of course  impedance control!).  Oops,

sorry for the unrequested speech on Cartool, you heard about it at the

presentation... anyways, if you're interested in it, let me know...

Here is the part of the Cartool main loop that does the impedance

control (I've added comments so you can follow what's happening) (the

_tw variables are twists (structure) that give information in the

three translations x, y and z, and thre rotations, around x, y, and z)

/******************************************************************************/

if (cart_modes.compliance_on)     /* if in compliant mode */

{

/* estimate actual cartesian velocity (we don't have joint

velocities): spring_position_tw: current cartesian position

ax_tw: old cartesian position (previous iteration)

aax_tw: old old cartesian position (second previous)

the algorithm was taken from some numerical analysis book, to

get efficient differentiation of the position */

dxest_tw = twist_scale(0.5/TgPeriod,

add_twist(

aax_tw,

sub_twist(twist_scale(3, spring_position_tw),

twist_scale(4, ax_tw))));

/* estimated acceleration (this term is not used in practice,

inertia_tw is set to zero, I just tried this once with no great success)*/

ddxest_tw = twist_scale(1.0/TgPeriod/TgPeriod,

add_twist(aax_tw, sub_twist(spring_position_tw,

twist_scale(2, ax_tw))));

/* global velocity demand for this iteration */

velocity_tw =

add_twist(add_twist(

prog_tw, /* commanded velocities: from the

trajectory generator or specific

user requests */

/* velocity correction factor */

mult_twist(cart_vars.cv_tw,

sub_twist(velocity_tw, dxest_tw))),

/* inputs from the trackball (velocities), only in

activated directions */

add_twist(mult_twist(track_disp_tw, cart_vars.track_input),

/* impedance control terms (equation 8 of paper) */

mult_twist(

sub_twist(

force_tw,

add_twist(

mult_twist(

cart_vars.stiffness_tw,

spring_position_tw),

mult_twist(

cart_vars.inertia_tw,

ddxest_tw))),

cart_vars.inverse_damping_tw)));

}

else

velocity_tw = prog_tw; /* if not in compliant mode, just used commanded trajectory inputs */

/* To prevent a too large velocity demand */

cartVelocityLimit(velocity_tw);

aax_tw = ax_tw;

ax_tw = spring_position_tw;

if (twist_norm_2(velocity_tw) > ZERO_VELOCITY)

{

/* convert displacement for this iteration to a transform */

tracking_disp_tr = twist_to_tr(twist_scale(TgPeriod, velocity_tw));

/* add transform to actual position, depending on the move mode */

switch (cart_modes.move_mode) {

case TOOL_MODE:

c_inv = tr_mult(c_inv, tracking_disp_tr);

break;

case BASE_MODE:

c_inv = tr_mult(tracking_disp_tr, c_inv);

break;

case FIXED_MODE:

c_inv = tr_mult2(fixed_tr, tracking_disp_tr, fixed_inv_c_inv_tr);

break;

case WORLD_MODE:

c_inv = tr_mult2(cart_frames.world_frame_tr, tracking_disp_tr, world_inv_c_inv_tr);

break;

case DECOUPLED_BASE_MODE:

c_inv = tr_mult3(tr_get_trans(tracking_disp_tr),

tr_get_trans(c_inv),

tr_get_rot(tracking_disp_tr),

tr_get_rot(c_inv));

break;

case DECOUPLED_FIXED_MODE:

c_inv = tr_mult(fixed_tr,

tr_mult3(tr_get_trans(tracking_disp_tr),

tr_get_trans(fixed_inv_c_inv_tr),

tr_get_rot(tracking_disp_tr),

tr_get_rot(fixed_inv_c_inv_tr)));

break;

case DECOUPLED_WORLD_MODE:

c_inv = tr_mult(cart_frames.world_frame_tr,

tr_mult3(tr_get_trans(tracking_disp_tr),

tr_get_trans(world_inv_c_inv_tr),

tr_get_rot(tracking_disp_tr),

tr_get_rot(world_inv_c_inv_tr)));

break;

default:

giveup("*** ERROR: can not move because move mode incorrectly set!!! ***");

break;

}

...

/******************************************************************************/

As you can see, the discrete-time implementation I used is very

simple.  Also, I don't use a damping variable br, but rather an

inverse damping 1/br (inverse_damping_tw).  In the case of no

compliance, i.e. br = infinity, I simply set 1/br to zero.

This scheme can easily be unstable (as you will certainly find out) if

1/br is set too high, especially in contact with stiff surfaces.  This

effect gets worse if the velocity correction factor is used. Typical

values I use (for 1/br) range from 0.00005 to 0.001.  With this algorithm, if

you leave the stiffness term to zero, and have some damping

compliance, the robot will tend to drift due to force sensor noise.

It is useful to have a function to reset the force sensor bias.

There, that's how I do it... hope this helps!  Please let me know if

you use this and how its going... do not hesitate if you

want more info also.  Good luck!

(From what I saw at the 1994 ICRA conference, everyone (or almost) that is doing

impedance control on position controlled robots is doing it this way,

i.e. based on the velocity demand... if you're not sure what method to

use, this one is in my opinion your best bet!)

Oh, by the way, I sent a copy of our annual video (the one you saw at

my presentation) to Clarence De Silva.  If you think anyone else can be

interested, do not hesitate to ask him! (In sent the "UBC copy" to him

since he's is the same IRIS-II project as me)

A la prochaine!

Michel

----------------------------------------------------------------------

Michel Pelletier

Institut de recherche d'Hydro-Quebec

Varennes, Quebec, Canada   J3X 1S1

Tel: (514) 652-8940

Fax: (514) 652-1316

email: michel@ireq-robot.hydro.qc.ca

----------------------------------------------------------------------