\section{Installation for Simulator-only Systems}

\begin{enumerate}

\item

{\it Build RCCL distribution --}

See Section \ref{installation.sec}.

Unpack tape, install GNU {\tt make}, edit the file {\ff conf/site.conf},

and run ``{\tt make World}''. Install the simulator network service.

\item

{\it Configure RCI for each robot --}

For each robot, give it a name and an entry in

{\ff robots/robotDataTab.c} (Section \ref{robotdatatab.sec})

and {\ff conf/robots} (Section \ref{robotsfile.sec}).

Create a {\tt .jls}, {\tt .kyn}, and {\tt .pos} file for each robot

(sections \ref{jlsfile.sec}, \ref{kynfile.sec}, and \ref{posfile.sec}).

\item

{\it Test the robot connections --}

Start the simulator program {\tt robotsim} with the name

of the robot you want to simulate. Try to ``move'' this

robot using the command {\tt move} with the {\tt -sim}

option.

\end{enumerate}